SIMH Users’ Guide, V4.0

18-Nov-2015
COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2012, written by Robert M Supnik

Copyright (c) 1993-2012, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

4Introduction


41
Compiling And Running A Simulator


51.1
Compiling Under UNIX/Linux/Mac OS-X


61.2
Compiling Under Windows


61.2.1
Compiling with Ethernet Support


61.2.2
Compiling Under MinGW


61.2.3
Compiling Under Visual C++


71.3
Compiling Under OpenVMS


82
Simulator Conventions


83
Commands


83.1
Loading and Saving Programs


83.2
Saving and Restoring State


93.3
Resetting Devices


93.4
Connecting and Disconnecting Devices


103.5
Examining and Changing State


123.6
Evaluating Instructions


123.7
Running A Simulated Program


123.7.1
Controlling The Simulation Rate


133.8
Stopping The Simulator


133.8.1
Simulator Detected Stop Conditions


133.8.2
User Specified Stop Conditions


133.8.3
Breakpoints


143.9
Halt on Output Data


143.9.1
Switches


153.10
Injecting Input Data


163.10.1
Delay


163.10.2
After


163.10.3
Escaping String Data


173.11
Setting Device Parameters


173.12
Displaying Parameters and Status


183.13
Altering The Simulated Configuration


183.14
Console Options


203.15
Remote Console


213.15.1
Master Mode


213.16
Executing Command Files


223.16.1
Displaying Arbitrary Text


223.16.2
Testing Simulator State


233.16.3
Example Test of Simulator State


243.17
Executing System Commands


243.18
Getting Help


243.19
Controlling Debugging


253.20
Exiting The Simulator


25Appendix 1: File Representations


25A.1 Hard Disks


25A.2 Floppy Disks


25A.3 Magnetic Tapes


26A.4 Line Printers


26A.5 DECtapes


26Appendix 2: Debug Status


29Revision History (covering Rev 2.0 to Rev 3.5)


35Acknowledgements




Introduction

This memorandum documents the SIMH simulators.  These simulators are freeware; refer to the license terms above for conditions of use.  Support is not available.  The best way to fix problems or add features is to read and modify the sources yourself.  Alternately, you can send Internet mail to simh AT trailing-edge DOT com, but a response is not guaranteed.

The simulators use a common command interface.  This memorandum describes the features of the command interface.  The details of each simulator are documented in separate, machine-specific memoranda.

1 Compiling And Running A Simulator

The simulators have been tested on VAX VMS, Alpha VMS, IA64 VMS, Alpha UNIX, NetBSD, FreeBSD, OpenBSD, Linux, Solaris, Windows 9x/NT/2000/XP/Win7/Win8, MacOS X, and OS/2.  Porting to other environments will require changes to the operating system dependent code in the SIMH libraries (sim_fio.c, sim_timer.c, sim_console.c, sim_ether.c, sim_sock.c, sim_disk.c, sim_serial.c).

The simulator sources are provided in a zip archive and are organized hierarchically.  Source files for the simulator libraries are in the top level directory; source files for each simulator are in individual subdirectories.  Note that the include files in the top level directory are referenced from the subdirectories, without path identifiers.  Your build tool needs to search the top level directory for include files not

present in the simulator-specific directory.  File manifests for each simulator are given in that simulator's documentation.

The simulators recognize or require a few compile-time #defines:

· The 18b simulators require that the model name be defined as part of the compilation command line (i.e., PDP4 for the PDP-4, PDP7 for the PDP-7, PDP9 for the PDP-9, PDP15 for the PDP-15).

· The PDP-10 and IBM 7094 simulators use 64b integer variables, requiring that USE_INT64 be defined as part of the compilation command line.  Since 64b integer declarations vary,  sim_defs.h has conditional declarations for Windows (_int64) and Digital UNIX (long).  The default is GNU C (long long).  If your compiler uses a different convention, you will have to modify sim_defs.h.

· The PDP-10, PDP-11, and VAX simulators share common peripherals.  To distinguish the target system, one of three variables must be defined on the command line: VM_PDP10 for the PDP-10; VM_PDP11 for the PDP-11; or VM_VAX for the VAX.

· The PDP-11, and VAX simulators optionally support Ethernet.  To include Ethernet emulation, USE_NETWORK must be defined as part of the compilation command line.  At present, Ethernet support has been tested only on Windows, Linux, OSX, NetBSD, OpenBSD, FreeBSD, Solaris, and Alpha VMS, but it should work in any host environment that supports the Pcap library (see the Ethernet readme file).

· The PDP-11 and VAX simulators optionally support disks and sequential tape files greater than 2GB.  To include large device support, both USE_INT64 and USE_ADDR64 must be defined as part of the compilation command line.

· The HP2100 Fast FORTRAN Processor (FFP) and 1000-F CPU options require 64b integer support.  Define HAVE_INT64 (not USE_INT64) as part of the compilation command line if your host compiler supports 64b integers.  On systems without 64b support, the 1000 F-Series CPU will be unavailable, and FFP extended-precision instructions (e.g., XADD) will be disabled; the remainder of the FFP instructions will work normally.  There may be some compilation warnings. 

To start the simulator, simply type its name.  (On VMS the simulators must then be defined as foreign commands in order to be started by name.)  The simulator recognizes three command line switches: -q, -v, and -e. If -q is specified, certain informational messages are suppressed.  The -v and -e switches pertain only to command files and are described in Section 3.13.

The simulator interprets the arguments on the command line, if any, as the file name and arguments for a DO command:


% pdp10 {switches} {<startup file> {arg,arg,...}}

If no file is specified on the command line, the simulator looks for a startup file consisting of the simulator name (including its path components) plus the extension .ini.  If a startup file is specified, either on the command line or implicitly via the .ini capability, it should contain a series of non-interactive simulator command, one per line.  These commands can be used to set up standard parameters, for example, disk sizes.

After initializing its internal structures and processing the startup file (if any), the simulator types out its name and version and then prompts for input with:


sim>

1.1 Compiling Under UNIX/Linux/Mac OS-X

The sources originate on a Windows system and have cr-lf at the end of every line.  For use on UNIX or Mac, the sources must be converted to UNIX or Mac text conventions.  This can be done with the UNZIP utility (unzip -a).

The supplied makefile will compile the simulators for UNIX systems that support the POSIX TERMIOS.  The VAX and PDP-11 can be compiled with or without Ethernet support.  The makefile will automatically build these simulators with Ethernet support if the necessary libpcap components are available on the system which is doing the building.  The recommended libpcap components are those packaged and provided by the host operating system vendor.  If your host operating system vendor doesn’t distribute libpcap components (all modern ones do), you may be able to use the libpcap which can be downloaded and built from sources from http://www.tcpdump.org/. 

To compile with or without Ethernet support:


gmake {target|ALL|clean}

Notes for hand compilation:

· The default UNIX terminal handling model is the POSIX TERMIOS interface, which is supported by Linux, Mac OS/X, and Alpha UNIX.  If your UNIX only supports the BSD terminal interface, BSDTTY must be defined as part of the compilation command line.

· The PDP-8, PDP-11, 18b PDP, PDP-10, and Nova simulators use the math library.  If your UNIX does not link the math library automatically, you must add -lm to the compilation command line.

Examples:

· PDP-11 under TERMIOS UNIX:

% cc -DVM_PDP11 pdp11_*.c scp.c sim_*.c -lm -o pdp11

· PDP-9 under TERMIOS UNIX:

% cc -DPDP9 pdp18b_*.c scp.c sim_*.c -lm -o pdp9

· PDP-10 under BSD terminal UNIX:


% cc -DVM_PDP10 -DUSE_INT64 -DBSDTTY pdp10_*.c scp.c sim_*.c -lm -o pdp10

1.2 Compiling Under Windows

1.2.1 Compiling with Ethernet Support

The Windows-specific Ethernet code uses the WinPCAP 4.x package.  This package for Windows provides the libpcap package that is freely available for Unix systems.  Building simulators with built-in Ethernet support can be done if the required WinPcap components are available on your build system at compile time.  These components are available by downloading the file: 

https://github.com/simh/windows-build/archive/windows-build.zip
This zip file contains a file called README.md which explains how to locate the unpacked zip file contents to build simulators with Ethernet support.

In order for a simulator (built with Ethernet support) to provide working Ethernet functionality at run time, WinPCAP must be installed on the system.  In order to install the WinPcap package the following should be performed:

· Download V4.x from http://www.winpcap.org.

· Install the package as directed.

1.2.2 Compiling Under MinGW

MinGW (Minimalist GNU for Windows) is a free C compiler available from http://www.mingw.org.  Msys is a minimal set of Unix utilities to support Mingw, also available from http://www.mingw.org.  The distribution includes a batch file (build_mingw.bat) that will build all the simulators from source.  By default, the PDP-11 and VAX family simulators are built with Ethernet support if the necessary Winpcap components are available at build time

1.2.3 Compiling Under Visual C++

Visual C++ requires projects to be defined for each executable which is being built.  You can define your own project for each simulator you are interested in, or you can use the predefined project definitions for this simulator release.

1.2.3.1 Using the predefined project definitions

The current simh source includes a directory “Visual Studio Projects” which contains the visual studio projects for all supported simulators.  This directory contains a file named 0ReadMe_Projects.txt which explains how to locate simh source files with respect to the related required build components provide in the windows_build.zip file mentioned above in “Compiling with Ethernet Support”.

These projects produce executables in the BIN\NT\Win32-Debug or BIN\NT\Win32-Release directories.  All intermediate build files are kept in BIN\NT\Project\{simulator-name} directories.

1.2.3.2 Defining your own Visual C++ project definitions

Each simulator must be organized as a separate Visual C++ project.  Starting from an empty console application,

· Add all the files from the simulator file manifest to the project.

· Open the Project Settings (VC++ 98) or Properties (VC++ .NET) dialog box.

· Under C/C++, Category: General, add any required preprocessor definitions (for example, USE_INT64).

· Under C/C++, Category: Preprocessor, add the top level simulation directory to the Additional Include Directories.  For the VAX and PDP-10, you must also add the PDP-11 directory.

· Under Link, add wsock32.lib and winmm.lib at the end of the list of Object/Module Libraries.

· If you are building the PDP-11 and VAX with Ethernet support, you must also add the WinPCAP libraries (packet.lib, wpcap.lib) to the list of Object/Module libraries.

If you are using Visual C++ .NET, you must turn off /Wp64 (warn about potential 64b incompatibilities) and disable Unicode processing.  You will also have to turn off warning 4996 (“deprecated” string functions), or lower the warning level to /W1.  Otherwise, the compilations will generate a lot of spurious conversion warnings.

Alternatively, you can ‘start’ from a preexisting Visual Studio project which may have similar components to the project you are trying to create.  Starting from such a project is done by copying the existing .vcproj file to a project file with a new name.  The MOST IMPORTANT step of this process is to assure that the copy of the project file as a unique GUID.  Generate a new GUID by invoking the guidgen.exe program from the Visual Studio directories (C:\Program Files (x86)\Microsoft Visual Studio 9.0\Common7\Tools\guidgen.exe).  Click on the Copy button to get the newly generated GUID into the clipboard.  Open the target project file in a text editor (notepad) and replace the ProjectGUID value with the clipboard contents.  Replace the Name value (PDP1 for example) with the name of your new simulator.  Carefully do a global replace of all instances of the prior Name value (PDP1 in this case) with the name of your new simulator.  Save the project file.  The next step is to add the project file to the existing simh solution.  Open the simh.sln with visual studio and in the Solution Explorer pane, right click on the Solution Simh and select Add->Existing Project.  Browse and select your new project file.  All other changes to the project file (the particular source and include files which make up your simulator) can be added or adjusted here.

1.3 Compiling Under OpenVMS

Compiling on OpenVMS requires DEC C.  The simulators that require 64b (PDP-10 and VAX) will not compile on OpenVMS/VAX.  The SIMH distribution includes an MMS command file descrip.mms that will build all the simulators from source.  An example of hand compilation:

· PDP-8 under VMS:

$ cc scp.c,sim_*.c,[.pdp8]pdp8*.c

$ link/exec=pdp8 scp.obj,sim_*.obj,[.pdp8]pdp8*.obj

2 Simulator Conventions

A simulator consists of a series of devices, the first of which is always the CPU.  A device consists of named registers and one or more numbered units.  Registers correspond to device state, units to device address spaces.  Thus, the CPU device might have registers like PC, ION, etc, and a unit corresponding to main memory; a disk device might have registers like BUSY, DONE, etc, and units corresponding to individual disk drives.  Except for main memory, device address spaces are simulated as unstructured binary disk files in the host file system.  The SHOW CONFIG command displays the simulator configuration.

A simulator keeps time in terms of arbitrary units, usually one time unit per instruction executed.  Simulated events (such as completion of I/O) are scheduled at some number of time units in the future.  The simulator executes synchronously, invoking event processors when simulated events are scheduled to occur.  Even asynchronous events, like keyboard input, are handled by polling at synchronous intervals.  The SHOW QUEUE command displays the simulator event queue.

3 Commands

Simulator commands consist of a command verb, optional switches, and optional arguments.  Switches take the form:


-<letter>{<letter>...}

Multiple switches may be specified separately or together: -abcd and -a -b -c -d are treated identically.  Verbs, switches, and other input (except for file names) are case insensitive.

Any command beginning with semicolon (;) is considered a comment and ignored.

3.1 Loading and Saving Programs

The LOAD command (abbreviation LO) loads a file in binary loader format:


load <filename> {implementation options}

The types of formats supported are implementation specific.  Options (such as load within range) are also implementation specific.

The DUMP command (abbreviation DU) dumps memory in binary loader format:


dump <filename> {implementation options}

The types of formats supported are implementation specific.  Options (such as dump within range) are also implementation specific.

3.2 Saving and Restoring State

The SAVE command (abbreviation SA) save the complete state of the simulator to a file.  This includes the contents of main memory and all registers, and the I/O connections of devices:


save <filename>

The RESTORE command (abbreviation REST, alternately GET) restores a previously saved simulator state:


restore {-d} {-f} {-q} <filename>

Notes: 

1) SAVE file format compresses zeroes to minimize file size.

2) The simulator can’t restore active incoming telnet sessions to multiplexer devices, but the listening ports will be restored across a save/restore.

3) –d switch avoids detaching and reattaching devices during a restore

4) –f switch overrides the date timestamp check for attached files during a restore

5) –q suppresses warning messages about save file version information

3.3 Resetting Devices

The RESET command (abbreviation RE) resets a device or the entire simulator to a predefined condition.  If switch -p is specified, the device is reset to its power-up state:


RESET




reset all devices


RESET -p



powerup all devices


RESET ALL



reset all devices


RESET <device>


reset specified device

Typically, RESET stops any in-progress I/O operation, clears any interrupt request, and returns the device to a quiescent state.  It does not clear main memory or affect I/O connections.

3.4 Connecting and Disconnecting Devices

Except for main memory and network devices, units are simulated as unstructured binary disk files in the host file system.  Before using a simulated unit, the user must specify the file to be accessed by that unit.  The ATTACH (abbreviation AT) command associates a unit and a file:


ATTACH <unit> <filename>

If the -n switch is specified when an attach is executed, a new file is created, and an appropriate message is printed.

If the file does not exist, and the -e switch was not specified, a new file is created, and an appropriate message is printed.  If the -e switch was specified, a new file is not created, and an error message is printed.

If the -r switch is specified, or the file is write protected, ATTACH tries to open the file read only.  If the file does not exist, or the unit does not support read only operation, an error occurs.  Input-only devices, such as paper-tape readers, and devices with write lock switches, such as disks and tapes, support read only operation; other devices do not.  If a file is attached read only, its contents can be examined but not modified. 

For simulated magnetic tapes, the ATTACH command can specify the format of the attached tape image file:


ATTACH -f <tape_unit> <format> <filename>

The currently supported tape image file formats are:


SIMH




SIMH simulator format


E11




E11 simulator format


TPC




TPC format


P7B




Pierce simulator 7-track format

The tape format can also be set with the SET command prior to ATTACH:


SET <tape_unit> FORMAT=<format>


ATT <tape_unit> <filename>

The format of an attached file can be displayed with the SHOW command:


SHOW <tape_unit> FORMAT

For Telnet-based terminal emulators, the ATTACH command associates the master unit with a TCP/IP port:


ATTACH <unit> <port>

The port is a decimal number between 1 and 65535 that is not used by standard TCP/IP protocols.

For Ethernet emulators, the ATTACH command associates the simulated Ethernet with a physical Ethernet device:


ATTACH <unit> <physical device name>

The DETACH (abbreviation DET) command breaks the association between a unit and a file, port, or network device:


DETACH ALL



detach all units


DETACH <unit>


detach specified unit

The EXIT command performs an automatic DETACH ALL.

3.5 Examining and Changing State

There are four commands to examine and change state:

· EXAMINE (abbreviated E) examines state

· DEPOSIT (abbreviated D) changes state

· IEXAMINE (interactive examine, abbreviated IE) examines state and allows the user to interactively change it

· IDEPOSIT (interactive deposit, abbreviated ID) allows the user to interactively change state

All four commands take the form


command {modifiers} <object list>

Deposit must also include a deposit value at the end of the command.

There are four kinds of modifiers: switches, device/unit name, search specifier, and for EXAMINE, output file.  Switches have been described previously.  A device/unit name identifies the device and unit whose address space is to be examined or modified.  If no device is specified, the CPU (main memory)is selected; if a device but no unit is specified, unit 0 of the device is selected.

The search specifier provides criteria for testing addresses or registers to see if they should be processed.  A specifier consists of a logical operator, a relational operator, or both, optionally separated by spaces.


{<logical op> <value>} <relational op> <value>

where the logical operator is & (and), | (or), or ^ (exclusive or), and the relational operator is = or == (equal), ! or != (not equal), >= (greater than or equal), > (greater than), <= (less than or equal), or < (less than).  If a logical operator is specified without a relational operator, it is ignored.  If a relational operator is specified without a logical operator, no logical operation is performed.  All comparisons are unsigned.

The output file modifier redirects command output to a file instead of the console.  An output file modifier consists of @ followed by a valid file name.

Modifiers may be specified in any order.  If multiple modifiers of the same type are specified, later modifiers override earlier modifiers.  Note that if the device/unit name comes after the search specifier, the search values will interpreted in the radix of the CPU, rather than of the device/unit.

The "object list" consists of one or more of the following, separated by commas:


register



the specified register


register[sub1-sub2]

the specified register array locations,







starting at location sub1 up to and







including location sub2


register[sub1/length]

the specified register array locations,







starting at location sub1 up to but







not including sub1+length


register[ALL]


all locations in the specified register







array


register1-register2

all the registers starting at register1







up to and including register2


address



the specified location


address1-address2


all locations starting at address1 up to







and including address2


address/length


all location starting at address up to







but not including address+length


STATE




all registers in the device


ALL




all locations in the unit

Switches can be used to control the format of display information:


-o




display as octal


-d




display as decimal


-h




display as hexadecimal

Simulators typically provide these additional switches for address locations:


-a




display as ASCII


-c




display as character string


-m




display as instruction mnemonic

...and accept symbolic input (see documentation with each simulator).

Examples:


ex 1000-1100


examine 1000 to 1100


de PC 1040



set PC to 1040


ie 40-50



interactively examine 40:50


ie >1000 40-50


interactively examine the subset





   

of locations 40:50 that are >1000


ex rx0 50060


examine 50060, RX unit 0


ex rx sbuf[3-6]


examine SBUF[3] to SBUF[6] in RX


de all 0



set main memory to 0


de &77>0 0



set all addresses whose low order





   

bits are non-zero to 0


ex -m @memdump.txt 0-7777
dump memory to file

Note: to terminate an interactive command, simply type a bad value (eg, XYZ) when input is requested.

3.6 Evaluating Instructions

The EVAL command evaluates a symbolic instruction and returns the equivalent numeric value.  This is useful for obtaining numeric arguments for a search command:


EVAL <expression>
3.7 Running A Simulated Program

The RUN command (abbreviated RU) resets all devices, deposits its argument (if given) in the PC, and starts execution.  If no argument is given, execution starts at the current PC.

The GO command does not reset devices, deposits its argument (if given) in the PC, and starts execution.  If no argument is given, execution starts at the current PC.

The CONT command (abbreviated CO) does not reset devices and resumes execution at the current PC.

The STEP command (abbreviated S) resumes execution at the current PC for the number of instructions given by its argument.  If no argument is supplied, one instruction is executed.

The BOOT command (abbreviated BO) resets all devices and bootstraps the device and unit given by its argument.  If no unit is supplied, unit 0 is bootstrapped.  The specified unit must be attached.

3.7.1 Controlling The Simulation Rate

By default, the simulator runs as fast as possible (although at lower than normal priority) and will consume all available processing resources on the host system.  This will raise power consumption (and the operating temperature) of many PC’s, and drain the battery of a laptop.

The SET THROTTLE command allows the user to reduce the effective execution rate to a specified number of instructions per second, or to a specified percentage of total host computing time:


SET THROTTLE xM



set execution rate to x mips


SET THROTTLE xK



set execution rate to x kips


SET THROTTLE x%



limit simulator to x% of host time


SET THROTTLE insts/delay

execute ‘insts’ instructions and 

then sleep for ‘delay’ milliseconds

Throttling is only available on host systems that implement a precision real-time delay function.

The SET NOTHROTTLE command turns off throttling.  The SHOW THROTTLE command shows the current settings for throttling.

Some simulators implement a different form of resource management called idling.  Idling suspends simulated execution whenever the program running on the simulator is doing nothing, and runs the simulator at full speed when there is work to do.  Throttling and idling are mutually exclusive.

3.8 Stopping The Simulator

Programs run until the simulator detects an error or stop condition, or until the user forces a stop condition.

3.8.1 Simulator Detected Stop Conditions

These simulator-detected conditions stop simulation:

· HALT instruction.  If a HALT instruction is decoded, simulation stops.

· Breakpoint.  The simulator may support breakpoints (see below).

· I/O error.  If an I/O error occurs during simulation of an I/O operation, and the device stop-on-I/O-error flag is set, simulation usually stops.

· Processor condition.  Certain processor conditions can stop simulation; these are described with the individual simulators.

3.8.2 User Specified Stop Conditions

Typing the interrupt character stops simulation.  The interrupt character is defined by the WRU (where are you) console option and is initially set to 005 (^E).

3.8.3 Breakpoints

A simulator may offer breakpoint capability.  A simulator may define breakpoints of different types, identified by letter (for example, E for execution, R for read, W for write, etc).  At the moment, most simulators support only E (execution) breakpoints.

Associated with a breakpoint are a count and, optionally, one or more actions.  Each time the breakpoint is taken, the associated count is decremented.  If the count is less than or equal to 0, the breakpoint occurs; otherwise, it is deferred.  When the breakpoint occurs, the optional actions are automatically executed.

A breakpoint is set by the BREAK command:


BREAK {-types} {<addr range>{[count]},{addr range...}}{;action;action...}

If no type is specified, the simulator-specific default breakpoint type (usually E for execution) is used.  If no address range is specified, the current PC is used.  As with EXAMINE and DEPOSIT, an address range may be a single address, a range of addresses low-high, or a relative range of address/length.  Examples:


BREAK




set E break at current PC

BREAK -e 200


set E break at 200


BREAK 2000/2[2]


set E breaks at 2000,2001 with count = 2


BREAK 100;EX AC;D MQ 0

set E break at 100 with actions EX AC and

 D MQ 0


BREAK 100;



delete action on break at 100

Currently set breakpoints can be displayed with the SHOW BREAK command:


SHOW {-types} BREAK {ALL|<addr range>{,<addr range>...}}

Locations with breakpoints of the specified type are displayed.

Finally, breakpoints can be cleared by the NOBREAK command.

3.9 Halt on Output Data

In addition to breakpoints (halting the simulator on specific addresses), the simulator can halt when a

specific string has been output to the console. The EXPECT command provides a way to define a rule which will stop execution and take actions when specific output has been generated by the simulated system.


EXPECT {dev:line} {HALTAFTER=n,}”<string>” {actioncommand {; actioncommand} …}

<dev:line> specifies a particular device a line in that simulated device.  If it is not specified, the simulated system’s console device is the default device.

The string argument must be delimited by quote characters.  Quotes may be either single or double but the opening and closing quote characters must match.  Data in the string may contain escaped character strings.

When expect rules are defined, they are evaluated against recently produced output as each character is output to the device.  Since this evaluation processing is done on each output character, rule matching is not specifically line oriented.  If line oriented matching is desired, then rules should be defined which contain the simulated system’s line ending character sequence (i.e. “\r\n”).

Once data has matched any expect rule, that data is no longer eligible to match other expect rules which may already be defined.  Data which is output prior to the definition of an expect rule is not eligible to be matched against.

3.9.1 Switches


Switches can be used to influence the behavior of EXPECT rules

3.9.1.1 –p

Expect rules default to be one shot activities.  That is, a rule is automatically removed when a match occurs unless the rule is designated as a persistent rule by using a –p switch when the rule is defined.

3.9.1.2 –c

If an expect rule is defined with the –c switch, it will cause all pending expect rules on the current device to be cleared when the rule matches data in the device output stream.

3.9.1.3 –r

If an expect rule is defined with the –r switch, the string is interpreted as a regular expression applied to the output data stream.  This regular expression may contain parentheses delimited sub-groups.

The syntax of the regular expressions available are those supported by the Perl Compatible Regular Expression package (aka PCRE).  As the name implies, the syntax is generally the same as Perl regular expressions.  See http://perldoc.perl.org/perlre.html for more details.

If the PCRE package isn’t available in the environment when a simulator is built, the local system’s regular expression package (if available) is used and the regular expression syntax is limited to what may be provided there.

3.9.1.4 –i

If a regular expression expect rule is defined with the –i switch, character matching for that expression will be case independent.  The –i switch is only valid for regular expression expect rules (-r).

3.9.1.5 Escaping String Data

The following character escapes are explicitly supported when expect rules are defined NOT using regular expression match patterns:

\r
Expect the ASCII Carriage Return character (Decimal value 13)

\n 
Expect the ASCII Linefeed character (Decimal value 10)

\f 
Expect the ASCII Formfeed character (Decimal value 12)

\t 
Expect the ASCII Horizontal Tab character (Decimal value 9)

\v 
Expect the ASCII Vertical Tab character (Decimal value 11)

\b 
Expect the ASCII Backspace character (Decimal value 8)

\\ 
Expect the ASCII Backslash character (Decimal value 92)

\' 
Expect the ASCII Single Quote character (Decimal value 39)

\" 
Expect the ASCII Double Quote character (Decimal value 34)

\? 
Expect the ASCII Question Mark character (Decimal value 63)

\e 
Expect the ASCII Escape character (Decimal value 27)

as well as octal character values of the form:

\n{n{n}} where each n is an octal digit (0-7)

and hext character values of the form:

\xh{h} where each h is a hex digit (0-9A-Fa-f)

3.10 Injecting Input Data

The SEND command provides a way to insert input data in to the console device of a simulated system as if it were entered by a user:


SEND {AFTER=n,} {DELAY=n,}”<string>”

The string argument must be delimited by quote characters.  Quotes may be either single or double but the opening and closing quote characters must match.  Data in the string may contain escaped character strings.

The SEND command can also insert input into any serial device on a simulated system as if it was entered by a user.


SEND <dev>:line {AFTER=n,}{DELAY=n,}”<string>”

3.10.1 Delay

Specifies a positive integer representing a minimal instruction delay between characters being sent.   The value specified in a delay argument persists across SEND commands to the same device (console or serial device).  The delay parameter can be set by itself with:


SEND {<dev>:line} {DELAY=n}

The default value of the delay parameter is 1000.

3.10.2 After

Specifies a positive integer representing a minimul number of instructions which must execute before the first character in the string is sent.  The value specified as the after parameter persists across SEND commands to the same device (console or serial device).  The after parameter value can be set by itself with:


SEND {<dev>:line} {AFTER=n}

If the after parameter isn’t explicitly set, it defaults to the value of the delay parameter.

3.10.3 Escaping String Data

The following character escapes are explicitly supported:

\r
Sends the ASCII Carriage Return character (Decimal value 13)

\n 
Sends the ASCII Linefeed character (Decimal value 10)

\f 
Sends the ASCII Formfeed character (Decimal value 12)

\t 
Sends the ASCII Horizontal Tab character (Decimal value 9)

\v 
Sends the ASCII Vertical Tab character (Decimal value 11)

\b 
Sends the ASCII Backspace character (Decimal value 8)

\\ 
Sends the ASCII Backslash character (Decimal value 92)

\' 
Sends the ASCII Single Quote character (Decimal value 39)

\" 
Sends the ASCII Double Quote character (Decimal value 34)

\? 
Sends the ASCII Question Mark character (Decimal value 63)

\e 
Sends the ASCII Escape character (Decimal value 27)

as well as octal character values of the form:

\n{n{n}} where each n is an octal digit (0-7)

and hext character values of the form:

\xh{h} where each h is a hex digit (0-9A-Fa-f)

3.11 Setting Device Parameters

The SET command (abbreviated SE) changes the status of one or more device parameters:


SET <device> <parameter>{=<value},{<parameter>{=<value>},...}

or one or more unit parameters:


SET <unit> <parameter>{=<value>},{<parameter>{=<value>},...}

Most parameters are simulator and device specific.  Disk drives, for example, can usually be set WRITEENABLED or write LOCKED; if a device supports multiple drive types, the SET command can be used to specify the drive type.

All devices recognize the following parameters:


OCT




sets the data radix = 8


DEC




sets the data radix = 10


HEX




sets the data radix = 16

3.12 Displaying Parameters and Status

The SHOW command (abbreviated SH) displays the status of one or more device parameters:


SHOW {<modifiers} <device> <parameter>{=<value>},

{<parameter>{=<value>},...}

or one or more unit parameters:


SHOW {<modifiers} <unit> <parameter>{=<value>},



{<parameter>{=<value>},...}

There are two kinds of modifiers: switches and output file.  Switches have been described previously.  The output file modifier redirects command output to a file instead of the console.  An output file modifier consists of @ followed by a valid file name.

All devices implement parameters RADIX (the display radix), MODIFIERS (list of valid modifiers), and NAMES (logical name).  Other device and unit parameters are implementation-specific.

SHOW is also used to display global simulation state:

SHOW CONFIGURATION 

shows the simulator configuration and

the status of all devices and units

SHOW DEVICES


shows the simulator configuration

SHOW FEATURES


shows the simulator configuration with 

descriptions

SHOW MODIFIERS


shows all available modifiers

SHOW NAMES



show all logical names

SHOW QUEUE
shows the simulator event queue

SHOW TIME



shows the elapsed time since the last RUN

SHOW VERSION


show the simulator version and options

SHOW <device>


shows the status of the named device

SHOW <unit> 


shows the status of the named unit

SHOW THROTTLE


shows the current throttling mode 

SHOW SHOW



shows the show options for all devices 

SHOW ETHERNET


shows the status/availability of host 

Ethernet devices

SHOW SERIAL 


shows the status of host serial ports

SHOW MULTIPLEXER 


shows the status of all multiplexer devices

which have been attached

SHOW DEFAULT 


shows the current working directory

The SHOW DEVICES, SHOW CONFIGURATION, SHOW FEATURES will normally display all devices which exist in the simulator.  These commands can optionally only display the enabled devices when the –E switch is on the command line (i.e. SHOW –E DEVICES).

SHOW QUEUE and SHOW TIME display time in simulator-specific units; typically, one time unit represents one instruction execution.

3.13 Altering The Simulated Configuration

In most simulators, the SET <device> DISABLED command removes the specified device from the configuration.  A DISABLED device is invisible to running programs.  The device can still be RESET, but it cannot be ATTAChed, DETACHed, or BOOTed.  SET <device> ENABLED restores a disabled device to a configuration.

Most multi-unit devices allow units to be enabled or disabled:


SET <unit> ENABLED


SET <unit> DISABLED

When a unit is disabled, it will not be displayed by SHOW DEVICE.

The standard device names can be supplemented with logical names.  Logical names must be unique within a simulator (that is, they cannot be the same as an existing device name).  To assign a logical name to a device:


ASSIGN <device> <log-name>
assign log-name to device

To remove a logical name:


DEASSIGN <device>


remove logical name

To show the current logical name assignment:


SHOW <device> NAMES

show logical name, if any

To show all logical names:


SHOW NAMES

3.14 Console Options

Console options are controlled by the SET CONSOLE command.

The console terminal normally runs in the controlling window.  Optionally, the console terminal can be connected to a Telnet port.  This allows systems to emulate a VT100 using the built-in terminal emulation of the Telnet client.


SET CONSOLE TELNET=<port>
connect console terminal to Telnet session

on port


SET CONSOLE NOTELNET

disable console Telnet

Normally a console terminal configured to listen to on a telnet port requires that a telnet connection be active for the simulator to run.  A telnet console can have its contents written to a buffer and which will allow the simulator to run without an active telnet connection.  When a telnet connection is established, the buffer contents is presented to the telnet session and execution continues as if the connection had been there all along.  Console buffering can be enabled by the following command:


SET CONSOLE BUFFERED{=bufsiz}
enable console buffering and optionally

set the buffer size to ‘bufsiz’.  The default buffer size is 32768.


SET CONSOLE NOBUFFER

disable console buffering

Output to the console can be logged simultaneously to a file:


SET CONSOLE LOG=<filename>
log console output to file


SET CONSOLE NOLOG


disable logging

The console provides a limited key remapping capability:


SET CONSOLE WRU=<value>

interpret ASCII code value as WRU


SET CONSOLE BRK=<value>

interpret ASCII code value as BREAK

(0 disables)


SET CONSOLE DEL=<value>

interpret ASCII code value as DELETE


SET CONSOLE PCHAR=<value>
bit mask of printable characters in range

[31,0]

A simulator console can be connected to a serial port on the host system.


SET CONSOLE SERIAL=ser0

connect console to serial port ser0


SET CONSOLE SERIAL=COM1

connect console to serial port COM1


SET CONSOLE SERIAL=/dev/ttyS0
connect console to serial port /dev/ttyS0

The available Serial ports on the host system can be displayed with the command:


SHOW SERIAL



display available serial ports on host

Serial port speed, character size, parity and stop bits can be indicated on the by appending the speed, character size, parity and stop bits to the serial port name:


SET CONSOLE SERIAL=ser0;2400-8N1

This will connect at 2400 with 8 bit characters, no parity and 1 stop bit.  The default serial speed, character size, parity and stop bits is 9600-8N1.

Values are hexadecimal on hex CPU's, octal on all others.

The SHOW CONSOLE command displays the current state of console options:


SHOW CONSOLE


show all console options


SHOW CONSOLE TELNET

show console Telnet state


SHOW CONSOLE LOG


show console logging state


SHOW CONSOLE WRU


show value assigned to WRU


SHOW CONSOLE BRK


show value assigned to BREAK


SHOW CONSOLE DEL


show value assigned to DELETE


SHOW CONSOLE PCHAR

show value assigned to PCHAR

Both SET CONSOLE and SHOW CONSOLE accept multiple parameters, separated by commas, e.g.,


SET CONSOLE WRU=5,DEL=177
set code values for WRU and DEL

3.15 Remote Console

During simulator execution, it may occasionally be useful to enter commands to query or adjust some internal details of the simulator’s configuration or operation.  These activities could be achieved directly through the initiating session which started the simulator.  However, a simulator may be running in the context of a background process and not actually have an interactive interface to the initiating process, or it may be inconvenient to access the initiating process.  To support circumstances like this, a Remote Console facility can be enabled in the simulator.  The remote console capability is configured with the following commands:


SET REMOTE TELNET=<port>
enable remote console to connects via Telnet 

to port


SET REMOTE NOTELNET

disable remote console Telnet


SET REMOTE CONNECTIONS=n
specify the number of concurrent remote

console sessions available


SET REMOTE TIMEOUT=secs

specify the remote console idle command

timeout

The remote console configuration details can be viewed with the command:


SHOW REMOTE



display remote console configuration

The remote console facility has two modes of command input and execution:

Single Command Mode:  In single command mode you enter one command at a time and aren't concerned about what the simulated system is doing while you enter that command.  The command is executed once you've hit return.

Multiple Command Mode: In multiple command mode you initiate your activities by entering the WRU character (usually ^E).  This will suspend the current simulator execution.  You then enter commands as needed and when you are done you enter a CONTINUE command.  While entering Multiple Command commands, if you fail to enter a complete command before the timeout (specified by "SET REMOTE TIMEOUT=seconds"), a CONTINUE command is automatically processed and simulation proceeds.

A subset of normal simh commands are available for use in remote console sessions.

The Single Command Mode commands are: ATTACH, DETACH, PWD, SHOW, DIR, LS, ECHO, HELP

The Multiple Command Mode commands are: EXAMINE, IEXAMINE, DEPOSIT, EVALUATE, ATTACH, DETACH, ASSIGN, DEASSIGN, STEP, CONTINUE, PWD, SAVE, SET, SHOW, DIR, LS, ECHO, HELP

A remote console session will close when a EOF character is entered (i.e. ^D or ^Z).
3.15.1 Master Mode

Remote Console Master mode allows full control of simulator operation to performed over a remote TCP session.  This mode is potentially useful for applications like a simulated CPU front panel interface or remote debugging with a GDB “Serial Stub”.

Master mode is entered from a simulator command file with the following command:


SET REMOTE MASTER

enter master mode

Operation in Remote Console Master mode requires that the simulator’s console port be serviced via a telnet connect with a “SET CONSOLE TELNET=port” command.  Both a telnet connection to the console port AND to the Remote Console port is required be a simulator will enter Master Mode.

A Master Mode session can return control to the initiating simulator command with:


SET REMOTE NOMASTER

leave master mode

The Remote Console Master Mode commands are: EXAMINE, IEXAMINE, DEPOSIT, EVALUATE, ATTACH, DETACH, ASSIGN, DEASSIGN, STEP, CONTINUE, PWD, SAVE, CD, SET, SHOW, DIR, LS, ECHO, HELP, RUN, GO, BOOT, BREAK, NOBREAK, EXIT

3.16 Executing Command Files

The simulator can execute command files with the DO command:


DO <filename> {arguments...}
execute commands in file

The DO command allows command files to contain substitutable arguments.  The string %n, where n is between 1 and 9, is replaced with argument n from the DO command line. The string %0 is replaced with <filename>.  The sequences \% and \\ are replaced with the literal characters % and \, respectively.  Arguments with spaces can be enclosed in matching single or double quotation marks.

If the switch -v is specified, the commands in the file are echoed before they are executed.

If the switch -e is specified, command processing (including nested command invocations) will be aborted if a command error is encountered.  (Simulation stop never abort processing; use ASSERT to catch unexpected stops.)  Without the switch, all errors except ASSERT failures will be ignored, and command processing will continue.

DO commands may be nested up to ten invocations deep.

Several commands are particularly useful within command files.  While they may be executed interactively, they have only limited functionality when so used.

3.16.1 Displaying Arbitrary Text

The ECHO command is a useful way of annotating command files.  ECHO prints out its argument on the console:


ECHO <string>


output string to console

If there is no argument, ECHO prints a blank line on the console.  This may be used to provide spacing in the console display or log.

3.16.2 Testing Simulator State

The ASSERT command tests a simulator state condition expression and halts command file execution if the condition is false:


ASSERT <condition-expression>

If the result of the condition is false, an "Assertion failed" message is printed, and any running command file is aborted.  Otherwise, the command has no effect.

The IF command tests a simulator state condition expression and executes one or more commands if the condition is true:


IF <condition-expression> <action>{; <action>…}

If the result of the condition is true, the action command(s) are executed. Otherwise, the command has no effect.

3.16.2.1 Condition Expressions

3.16.2.1.1 Internal Simulator Variables Expressions

{NOT} {<dev>} <reg>{<logical-op><value>}<conditional-op><value>

If <dev> is not specified, CPU is assumed.  <reg> is a register (scalar or subscripted) belonging to the indicated device.  The <conditional-op> and optional <logical-op> are the same as those used for "search specifiers" by the EXAMINE and DEPOSIT commands (see above).  The <value>s are expressed in the radix specified for <reg>, not in the radix for the device.

If the <logical-op> and <value> are specified, the target register value is first altered as indicated.  The result is then compared to the <value> via the <conditional-op>.  If the NOT unary operator precedes the expression, the results value is inverted.  

3.16.2.1.2 String Comparison Expressions

String Values can be compared with:

{-i} {NOT} “<string1>” <compare-op> “<string2>”

The –I switch, if present, causes comparisions to be case insensitive. <string1> and <string2> are quoted string values which may have environment variables substituted as desired.  <compare-op> may be one of: 

==
- equal

EQU
- equal

!=
- not equal

NEQ
- not equal

<
- less than

LSS
- less than

<=
- less than or equal

LEQ
- less than or equal

>
- greater than

GTR
- greater than

>=
- greater than or equal

GEQ
- greater than or equal

Comparisons are generic.  This means that if both string1 and string2 are comprised of all numeric digits, then the strings are converted to numbers and a numeric comparison is performed.  For example: “+1” EQU “1” will be true. 

3.16.3 Example Test of Simulator State

A command file might be used to bootstrap an operating system that halts after the initial load from disk.  The ASSERT command is then used to confirm that the load completed successfully by examining the CPU's "A" register for the expected value:


; OS bootstrap command file


;


ATTACH DS0 os.disk


BOOT DS


; A register contains error code; 0 = good boot


ASSERT A=0


ATTACH MT0 sys.tape


ATTACH MT1 user.tape


RUN

In the example, if the A register is not 0, the "ASSERT A=0" command will be echoed, the command file will be aborted with an "Assertion failed" message.  Otherwise, the command file will continue to bring up the operating system.

Alternative, the IF command could be used to solve the same problem:


; OS bootstrap command file


;


ATTACH DS0 os.disk


BOOT DS


; A register contains error code; 0 = good boot


IF NOT A=0 ECHO Assertion Failed; EXIT AFAIL


ATTACH MT0 sys.tape


ATTACH MT1 user.tape


RUN

3.17 Executing System Commands

The simulator can execute operating system commands with the ! (spawn) command:


! <host operating system command>

If no operating system command is provided, the simulator attempts to launch the host operating system's command shell.

3.18 Getting Help

The HELP command prints out information about a command or about all commands:


HELP




print all HELP messages


HELP <command>


print HELP for command


HELP <device>


print HELP for device


HELP <device> REGISTERS

print HELP for device register variables


HELP <device> ATTACH

print HELP for device specific attach


HELP <device> SET


print HELP for device specific SET commands


HELP <device> SHOW

print HELP for device specific SHOW commands

3.19 Controlling Debugging

Some simulated devices may provide debug printouts to help in diagnosing complicated problems.  Debug output may be sent to a variety of places, or disabled entirely:


SET CONSOLE DEBUG=STDOUT
direct debug output to stdout


SET CONSOLE DEBUG=STDERR
direct debug output to stderr


SET CONSOLE DEBUG=LOG
  
direct debug output to log file


SET CONSOLE DEBUG=<filename>
direct debug output to file


SET CONSOLE NODEBUG
  
disable debug output

If debug output is enabled, individual devices can be controlled with the SET command.  If a device has only a single debug flag:


SET <device> DEBUG

enable device debug output


SET <device> NODEBUG
  
disable device debug output

If the device has individual, named debug flags:


SET <device> DEBUG

enable all debug flags


SET <device> DEBUG=n1;n2;...
enable debug flags n1, n2, ...


SET <device> NODEBUG=n1;n2;... disable debug flags n1, n2, ...


SET <device> NODEBUG

disable all debug flags

If debug output is directed to stdout, it will be intermixed with normal simulator output.

3.20 Exiting The Simulator

EXIT (synonyms QUIT and BYE) returns control to the operating system.

Appendix 1: File Representations

All file representations are little-endian.  On big-endian hosts, the simulator automatically performs any required byte swapping.

A.1 Hard Disks

Hard disks are represented as unstructured binary files of 16b data items for the 12b and 16b simulators, of 32b data items for the 18b, 24b, and 32b simulators, and 64b for the 36b simulators.  Device simulations which use the sim_disk library can also have hard disks which are Virtual Hard Disks (as described by the Microsoft Open Specification) and for Windows and Linux hosts can be raw host disks and/or CDROM devices.

A.2 Floppy Disks

Floppy disks are represented as unstructured binary files of 8b data items.  They are nearly identical to the floppy disk images for Doug Jones' PDP-8 simulator but lack the initial 256 byte header.  A utility for converting between the two formats is easily written.

A.3 Magnetic Tapes

Magnetic tapes are represented as unstructured binary files of 8b data items.  Each record starts with a 32b record header, in little endian format.  If the record header is not a special header, it is followed by n 8b bytes of data, followed by a repeat of the 32b record header.  A 1 in the high order bit of the record header indicates an error in the record. If the byte count is odd, the record is padded to even length; the pad byte is undefined.

Special record headers occur only once and have no data.  The currently defined special headers are:


0x00000000

file mark


0xFFFFFFFF

end of medium


0xFFFFFFFE

erase gap

Magnetic tapes are endian independent and consistent across simulator families.  A magnetic tape produced by the Nova simulator will appear to have its 16b words byte swapped if read by the PDP-11 simulator.

SIMH can read and write E11-format magnetic tape images.  E11 format differs from SIMH format only for odd-length records; the data portion of E11 records is not padded with an extra byte.

SIMH can read TPC-format magnetic tape images.  TPC format uses a 16b record header, with 0x0000 denoting file mark.  The record header is not repeated at the end of the record.  Odd-length records are padded with an extra byte.  Some TPC formatted tapes have an end of medium indicated as a record length of 0xffff with no data following the record length.

SIMH can read Pierce-format seven-track magnetic tape images.  Pierce format uses only 6 data bits, and one parity bit, in each byte.  The high order bit indicates start of record.  End of file is indicated by a record of one (occasionally two) bytes consisting of code 017 (octal).

A.4 Line Printers

Line printer output is represented by an ASCII file of lines separated by the newline character.  Overprinting is represented by a line ending in return rather than newline.

A.5 DECtapes

DECtapes are structured as fixed length blocks.  PDP-1/4/7/9/15 DECtapes use 578 blocks of 256 32b words.  Each 32b word contains 18b (6 lines) of data.  PDP-11 DECtapes use 578 blocks of 256 16b words.  Each 16b word contains 6 lines of data, with 2b omitted.  This is compatible with native PDP-11 DECtape dump facilities, and with John Wilson's PUTR Program.  PDP-8 DECtapes use 1474 blocks of 129 16b words.  Each 16b word contains 12b (4 lines) of data.  PDP-8 OS/8 does not use the 129th word of each block, and OS/8 DECtape dumps contain only 128 words per block.  A utility, DTOS8CVT.C, is provided to convert OS/8 DECtape dumps to simulator format.

A known issue in DECtape format is that when a block is recorded in one direction and read in the other, the bits in a word are scrambled (to the complement obverse).  The PDP-11 deals with this problem by performing an automatic complement obverse on reverse writes and reads.  The other systems leave this problem to software.

The simulator represents this difference as follows.  On the PDP-11, all data is represented in normal form.  Data reads and writes are not direction sensitive; read all and write all are direction sensitive.  Real DECtapes that are read forward will generate images with the correct representation of the data.

On the other systems, forward write creates data in normal form, while reverse write creates data in complement obverse form.  Forward read (and read all) performs no transformations, while reverse read (and read all) changes data to the complement obverse.  Real DECtapes that are read forward

will generate data in normal form for blocks written forward, and complement obverse data for blocks written in reverse, corresponding to the simulator format.

Appendix 2: Debug Status

The debug status of each simulated CPU and device is as follows:


System    PDP-8 PDP-11 Nova PDP-1 18b PDP

device

CPU


y
y
y
y
y

FPU


n
y
-
-
h

EIS/CIS

-
h
-
-
-

console

y
y
y
y
y

paper tape

y
y
y
y
y

card reader

-
y
-
-
-

line printer
y
y
y
h
y

clock


y
y
y
-
y

extra terminal
y
y
y
-
y

hard disk

y
y
y
-
y

fixed disk

y
y
h
-
y

floppy disk

y
y
y
-
-

drum


-
-
-
h
h

DECtape

y
y
-
h
y

mag tape

y
y
y
-
h


system    1401  2100  PDP-10 H316 MicroVAX 3900

device

CPU


y
y
y
h
y

FPU


-
y
y
-
y

EIS/CIS

-
y
y
-
-

console

h
y
y
h
y

paper tape

-
y
h
h
-

card reader

y
-
-
-
y

line printer
y
y
y
h
y

clock


-
y
y
h
y

extra terminal
-
y
y
-
y

hard disk

h
y
y
h
y

fixed disk

-
y
-
h
-

floppy disk

-
-
h
-
y

drum


-
y
-
-
-

DECtape

-
-
-
-
-

mag tape

y
y
y
h
y


system    GRI   1620   i16   i32  SDS940

device

CPU


h
h
d
y
d

FPU


-
h
d
y
-

CIS


-
-
-
-
-

console

h
h
d
y
h

paper tape

h
h
d
y
h

card reader

-
h
-
-
-

line printer
-
h
d
y
h

clock


h
-
d
y
n

extra terminal
-
-
h
y
h

hard disk

-
h
d
y
h

fixed disk

-
-
-
-
h

floppy disk

-
-
d
d
-

drum


-
-
-
-
h

DECtape

-
-
-
-
-

mag tape

-
-
d
y
h


system   LGP-30 1130   780

device

CPU


h
y
y

FPU


-
-
y

CIS


-
-
y

console

h
y
y

paper tape

h
-
-

card reader

-
y
y

line printer
-
y
y

clock


-
-
y

extra terminal
-
-
y

hard disk

-
y
y

fixed disk

-
-
-

floppy disk

-
-
y

drum


-
-
-

DECtape

-
-
-

mag tape

-
-
y

legend:

y = runs operating system or sample program




d = runs diagnostics




h = runs hand-generated test cases




n = untested




- = not applicable

Revision History (covering Rev 2.0 to Rev 3.5)

Starting with Rev 2.7, detailed revision histories can be found in file sim_rev.h.

Rev 3.5, Sep, 05

· Overhauled sources for readability

· Added VAX-11/780

Rev 3.4, May, 05

· Revised memory interaction model

Rev 3.3, Nov, 04

· Added PDP-11/VAX DHQ11 support

· Added PDP-11/VAX TM02/TM03 support

· Added PDP-11 model-specific emulation support

· Added full VAX support

· Replaced SET ONLINE/OFFLINE with SET ENABLED/DISABLED

Rev 3.2, Apr, 04

· Added LGP-30/LGP-21 simulator

· Added global SHOW modifier capability

· Added global SET DEBUG modifier

· Added global SHOW DEBUG,RADIX,MODIFIERS,NAME modifiers

· Added VAX extended physical memory support (Mark Pizzolato)

· Added VAX RXV21 support

· Revised terminal multiplexer library to support variable number of lines per multiplexer

· Added PDP-15 LT19 support (1-16 terminals)

Rev 3.1, Dec, 03

· Added Alpha/VMS, FreeBSD, Mac OS/X Ethernet library support

· Added Eclipse floating point and interval timer support (from Charles Owen)

· Added PDP-1 parallel drum support

· Added PDP-8 TSC8-75 and TD8E support

· Added H316/516 DMA/DMC, magtape, fixed head disk support

· Added PDP-8, PDP-15, 32b Interdata instruction history support

Rev 3.0, May, 03

· Added logical name support

· Added instruction history support

· Added multiple tape format support

· Added 64b address support

· Added PDP-4 EAE support

· Added PDP-15 FP15 and XVM support

Rev 2.10, Nov, 02

· Added Telnet console capability, removed VT emulation

· Added DO with substitutable arguments (from Brian Knittel)

· Added .ini initialization file (from Hans Pufal)

· Added quiet mode (from Brian Knittel)

· Added ! command (from Mark Pizzolato)

· Added Telnet BREAK support (from Mark Pizzolato)

· Added device enable/disable support

· Added optional simulator hooks for input, output, commands

· Added breakpoint actions

· Added magnetic tape simulation library

· Added PDP-11 KW11P programmable clock

· Added PDP-11 RK611/RK06/RK07 disk

· Added PDP-11/VAX TMSCP tape

· Added PDP-11/VAX DELQA Ethernet support (from David Hittner)

· Added PDP-11/PDP-10 RX211/RX02 floppy disk

· Added PDP-11/VAX autoconfiguration support

· Added PDP-10/PDP-11/VAX variable vector support

· Added PDP-1 DECtape

· Added PDP-1, PDP-4 Type 24 serial drum support

· Added PDP-8 RX28 support

· Added PDP-9 RB09 fixed head disk, LP09 line printer

· Added HP2100 12845A line printer

· Added HP2100 13183 magtape support

· Added HP2100 boot ROM support

· Added HP2100 interprocessor link support

· Added IBM 1620

· Added SDS 940

· Added Interdata 16b and 32b systems

· Added 16b DECtape file format support

· Added support for statically buffered devices

· Added magnetic tape end of medium support

· Added 50/60Hz support to line frequency clocks

· Added 7B/8B support to terminals and multiplexers

· Added BREAK support to terminals and multiplexers

Rev 2.9, Jan, 02

· Added circular register arrays

· Replaced ENABLE/DISABLE with SET ENABLED/DISABLED

· Replaced LOG/NOLOG with SET LOG/NOLOG

· Generalized the timer calibration package

· Added additional routines to the multiplexer library

· Added SET DISCONNECT, SHOW STATISTICS commands to multiplexers

· Re-implemented PDP-8 TTX as a unified multiplexer

· Implemented a PC queue in most simulators

· Added VAX simulator

· Added GRI-909 simulator

· Added Peter Schorn's MITS 8080/Z80 simulator

· Added Brian Knittel's IBM 1130 simulator

· Added HP2100 DQ, DR, MS, MUX devices

· Added SET VT/NOVT commands

Rev 2.8, Dec, 01

· Added DO command

· Added general breakpoint facility

· Added extended SET/SHOW capability

· Replaced ADD/REMOVE with SET ONLINE/OFFLINE

· Added global register name recognition

· Added unit-based register arrays

· Added Charles Owen's System 3 simulator

· Added PDP-11 I/O bus map

· Added PDP-11/VAX RQDX3

· Added PDP-8 RL8A

· Revised 18b PDP interrupt structure

· Revised directory and documentation structure

· Added support for MINGW environment

Rev 2.7, Sep, 01

· Added DZ11 (from Thord Nilson and Art Krewat) to PDP-11, PDP-10

· Added additional terminals to PDP-8

· Added TSS/8 packed character format to PDP-8

· Added sim_sock and sim_tmxr libraries

· Added sim_qcount and simulator exit detach all facilities

· Added Macintosh sim_sock support (from Peter Schorn)

· Added simulator revision level, SHOW version

· Changed int64/uint64 to t_int64/t_uint64 for Windows

· Fixed bug in PDP-11 interrupt acknowledge

· Fixed bugs in PDP-11 TS NXM check, boot code, error status; added extended characteristics and status

· Fixed bug in PDP-11 TC stop, stop all functions

· Fixed receive interrupt while disconnected bug in DZ11

· Fixed multi-unit operation bugs, interrupt bugs in PDP-11

· RP, PDP-10 RP, PDP-10 TU

· Fixed carrier detect bug in PDP-11, PDP-10 DZ

· Fixed bug in PDP-8 reset routine

· Fixed conditional in PDP-18b CPU

· Fixed SC = 0 bug in PDP-18b EAE

· Fixed bug in PDP-7 LPT

· Upgraded Nova second terminal to use sim_tmxr

· Upgraded PDP-18b second terminal to use sim_tmxr

· Upgraded PDP-11 LTC to full KW11-L

· Removed hack multiple console support

Rev 2.6b, Aug, 01

· Added H316/516 simulator

· Added Macintosh support from Louis Chrétien, Peter Schorn, and Ben Supnik

· Added bad block table option to PDP-11 RL, RP

· Removed register in declarations

· Fixed bugs found by Peter Schorn

· endian error in PDP-10, PDP-11 RP

· space reverse error in PDP-11 TS

· symbolic input in 1401

· Fixed bug in PDP-1 RIM loader found by Derek Peschel

· Fixed bug in Nova fixed head disk

Rev 2.6a, Jun, 01

· Added PDP-9, PDP-15 API option

· Added PDP-9, PDP-15 second terminal

· Added PDP-10 option for TOPS-20 V4.1 bug fix

· Added PDP-10 FE CTRL-C option for Windows

· Added console logging

· Added multiple console support

· Added comment recognition

· Increased size of string buffers for long path names

· Fixed bug in big-endian I/O found by Dave Conroy

· Fixed DECtape reset in PDP-8, PDP-11, PDP-9/15

· Fixed RIM loader PC handling in PDP-9/15

· Fixed indirect pointers in PDP-10 paging

· Fixed SSC handling in PDP-10 TM02/TU45

· Fixed JMS to non-existent memory in PDP-8

· Fixed error handling on command file

Rev 2.6, May, 01

· Added ENABLE/DISABLE devices

· Added SHOW DEVICES

· Added examination/modification of register arrays

· Added PDP-10 simulator

· Added clock autocalibration to SCP, Nova, PDP-8, PDP-11, PDP-18b

· Added PDP-8, PDP-11, PDP-9/15 DECtape

· Added PDP-8 DF32

· Added 4k Disk Monitor boot to PDP-8 RF08 and DF32

· Added PDP-4/7 funny format loader support

· Added extension handling to the PDP-8 and -9/15 loaders

· Added PDP-11 TS11/TSV05

· Added integer interval timer to SCP

· Added filename argument to LOAD/DUMP

· Revised magnetic tape and DECtape bootstraps to rewind before first instruction

· Fixed 3 cycle data break sequence in PDP-8 RF

· Fixed 3 cycle data break sequence in 18b PDP LP, MT, RF

· Fixed CS1.TRE write, CS2.MXF,UPE write, and CS2.UAI in PDP-11 RP

· Fixed 4M memory size definition in PDP-11

· Fixed attach bug in RESTORE

· Fixed detach bug for buffered devices

· Updated copyright notices, fixed comments

Rev 2.5a, Dec, 00

· Added CMD flop to HP paper tape and line printer

· Added status input for HP paper tape punch and TTY

· Added Charles Owen's 1401 mag tape boot routine

· Added Bruce Ray's Nova plotter and second terminal modules

· Added Charles Owen's Eclipse CPU support

· Added PDP-9/PDP-15 RIM/BIN loader support

· Added PDP-9/PDP-15 extend/bank initial state registers

· Added PDP-9/PDP-15 half/full duplex support

· Moved software documentation to a separate file

· Fixed SCP handling of devices without units

· Fixed FLG, FBF initialization in many HP peripherals

· Fixed 1401 bugs found by Charles Owen

· 4, 7 char NOPs are legal

· 1 char B is chained BCE

· MCE moves whole character, not digit, after first

· Fixed Nova bugs found by Bruce Ray

· traps implemented on Nova 3 as well as Nova 4

· DIV and DIVS 0/0 set carry

· RETN sets SP from FP at outset

· IORST does not clear carry

· Nova 4 implements two undocumented instructions

· Fixed bugs in 18b PDP's

· XCT indirect address calculation

· missing index instructions in PDP-15

· bank mode handling in PDP-15

Rev 2.5, Nov, 00

· Removed Digital and Compaq from copyrights, as authorized by Compaq Sr VP Bill Strecker

· Revised save/restore format for 64b simulators

· Added examine to file

· Added unsigned integer data types to sim_defs

· Added Nova 3 and Nova 4 instructions to Nova CPU

· Added HP2100

· Fixed indirect loop through autoinc/dec in Nova CPU

· Fixed MDV enabled test in Nova CPU

Rev 2.4, Jan, 99

· Placed all sources under X11-like open source license

· Added DUMP command, revised sim_load interface

· Added SHOW MODIFIERS command

· Revised magnetic tape format to include record error flag   

· Fixed 64b problems in SCP

· Fixed big endian problem in PDP-11 bad block routine 

· Fixed interrupt on error bug in PDP-11 RP/RM disks

· Fixed ROL/ROR inversion in PDP-11 symbolic routines

Rev 2.3d, Sep, 98

· Added BeOS support

· Added radix commands and switches

· Added PDP-11 CIS support

· Added RT11 V5.3 to distribution kits

· Fixed "shift 32" bugs in SCP, PDP-11 floating point

· Fixed bug in PDP-11 paper tape reader

· Fixed bug in ^D handling

Rev 2.3c, May, 98

· Fixed bug in PDP-11 DIV overflow check

· Fixed bugs in PDP-11 magnetic tape bootstrap

· Fixed bug in PDP-11 magnetic tape unit select

· Replaced UNIX V7 disk images

Rev 2.3b, May, 98

· Added switch recognition to all simulator commands

· Added RIM loader to PDP-8 paper tape reader and loader

· Added second block bootstrap to PDP-11 magnetic tape

· Fixed bug in PDP-8 RF bootstrap

· Fixed bug in PDP-11 symbolic display

· Fixed bugs in PDP-11 floating point (LDEXP, STEXP, MODf, STCfi, overflow handling)

Rev 2.3a, Nov, 97

· Added search capability

· Added bad block table command to PDP-11 disks

· Added bootstrap to PDP-11 magnetic tape

· Added additional Nova moving head disks

· Added RT-11 sample software

· Fixed bugs in PDP-11 RM/RP disks

· Fixed bugs in Nova moving head disks

· Fixed endian dependence in 18b PDP RIM loader

Rev 2.3, Mar, 97

· Added PDP-11 RP

· Added PDP-1

· Changed UNIX terminal I/O to TERMIOS

· Changed magnetic tape format to double ended

· Changed PDP-8 current page mnemonic from T to C

· Added endian independent I/O routines

· Added precise integer data types 

· Fixed bug in sim_poll_kbd

· Fixed bug in PDP-8 binary loader

· Fixed bugs in TM11 magnetic tape

· Fixed bug in RX11 bootstrap

· Fixed bug in 18b PDP ADD

· Fixed bug in 18b PDP paper tape reader

· Fixed bug in PDP-4 console

· Fixed bug in PDP-4, 7 line printer

Rev 2.2d, Dec, 96

· Added ADD/REMOVE commands

· Added unit enable/disable support to device simulators

· Added features for IBM 1401 project

· Added switch recognition for symbolic input

· Fixed bug in variable length IEXAMINE

· Fixed LCD bug in RX8E

· Initial changes for Win32

· Added IBM 1401

Rev 2.2b, Apr, 96

· Added PDP-11 dynamic memory size support

Rev 2.2a, Feb, 96

· New endian independent magnetic tape format

Rev 2.2 Jan, 96

· Added register buffers for save/restore

· Added 18b PDP's

· Guaranteed TTI, CLK times are non-zero

· Fixed breakpoint/RUN interaction bug

· Fixed magnetic tape backspace to EOF bug

· Fixed ISZ/DCA inversion in PDP-8 symbol table

· Fixed sixbit conversion in PDP-8 examine/deposit

· Fixed origin increment bug in PDP-11 binary loader

· Fixed GCC longjmp optimization bug in PDP-11 CPU

· Fixed unit number calculation bug in SCP and in Nova, PDP-11, 18b PDP moving head disks

Rev 2.1 Dec, 95

· Fixed PTR bug (setting done on EOF) in PDP-8, Nova

· Fixed RX bug (setting error on INIT if drive 1 is not attached) in PDP-8, PDP-11

· Fixed RF treatment of photocell flag in PDP-8

· Fixed autosize bug (always chose smallest disk if new file) in PDP-11, Nova

· Fixed not attached bug (reported as not attachable) in most mass storage devices

· Fixed Nova boot ROMs

· Fixed bug in RESTORE (didn't requeue if delay = 0)

· Fixed bug in RESTORE (clobbered device position)

· Declared static constant arrays as static const

· Added PDP-8, Nova magnetic tape simulators

· Added Dasher mode to Nova terminal simulator

· Added LINUX support

Rev 2.0
May, 95

· Added symbolic assembly/disassembly

Acknowledgements

SIMH would not have been possible without help from around the world.  I would like to acknowledge the help of the following people, all of whom donated their time and talent to this "computer archaeology" project:

Bill Ackerman

PDP-1 consulting

Alan Bawden

ITS consulting

Winfried Bergmann
Linux port testing

Phil Budne

Solaris port testing

Max Burnet

PDP information, documentation, and software

Robert Alan Byer
VMS socket support and testing 

James Carpenter
LINUX port testing

Chip Charlot

PDP-11 RT-11, RSTS/E, RSX-11M legal permissions

Louis Chrétien

Macintosh porting

Dave Conroy

HP 21xx documentation, PDP-10, PDP-18b debugging

L Peter Deutsch

PDP-1 LISP software

Ethan Dicks

PDP-11 2.9 BSD debugging

John Dundas

PDP-11 CPU debugging, programmable clock simulator

Jonathan Engdahl
PDP-11 device debugging

Carl Friend

Nova and Interdata documentation, and RDOS software

Megan Gentry

PDP-11 integer debugging

Dave Gesswein

PDP-8 and PDP-9.15 documentation, PDP-8 DECtape, disk, and paper-tape




 images, PDP-9/15 DECtape images

Dick Greeley

PDP-8 OS/8 and PDP-10 TOPS-10/20 legal permissions

Gordon Greene

PDP-1 LISP machine readable source

Lynne Grettum

PDP-11 RT-11, RSTS/E, RSX-11M legal permissions

Franc Grootjen

PDP-11 2.11 BSD debugging

Doug Gwyn

Portability debugging

Kevin Handy

TS11/TSV05 documentation, make file

Ken Harrenstein
KLH PDP-10 simulator

Bill Haygood

PDP-8 information, simulator, and software

Wolfgang Helbig
DZ11 implementation

Mark Hittinger

PDP-10 debugging

Dave Hittner

SCP debugging, DEQNA emulator and Ethernet library

Sellam Ismail

GRI-909 documentation

Jay Jaeger

IBM 1401 consulting

Doug Jones

PDP-8 information, simulator, and software

Brian Knittel

IBM 1130 simulator, SCP extensions for GUI support

Al Kossow

HP 21xx, Varian 620, TI 990, Interdata, DEC documentation and software

Arthur Krewat

DZ11 changes for the PDP-10

Mirian Crzig Lennox
ITS and DZ11 debugging

Don Lewine

Nova documentation and legal permissions

Tim Litt


PDP-10 hardware documentation and schematics, tape images, and




software sources

Tim Markson

DZ11 debugging

Bill McDermith

HP 2100 debugging, 12565A simulator

Scott McGregor

PDP-11 UNIX legal permissions

Jeff Moffatt

HP 2100 information, documentation, and software

Alec Muffett

Solaris port testing

Terry Newton

HP 21MX debugging

Thord Nilson

DZ11 implementation

Charles Owen

Nova moving head disk debugging, Altair simulator, Eclipse simulator,




 IBM System 3 simulator, IBM 1401 diagnostics, debugging, and magtape boot

Sergio Pedraja

MINGW environment debugging

Derek Peschel

PDP-10 debugging

Paul Pierce

IBM 1401 diagnostics, media recovery

Mark Pizzolato

SCP, Ethernet, and VAX simulator improvements

Hans Pufal

PDP-10 debugging, PDP-15 bootstrap, DOS-15 recovery, DOS-15 documentation,




 PDP-9 restoration

Bruce Ray

Software, documentation, bug fixes, and new devices for the Nova, OS/2 porting

Craig St Clair

DEC documentation

Richard Schedler
Public repository maintenance

Peter Schorn

Macintosh porting

Stephen Schultz
PDP-11 2.11 BSD debugging

Olaf Seibert

NetBSD port testing

Brian & Barry Silverman
PDP-1 simulator and software

Tim Shoppa

Nova documentation, RDOS software, PDP-10 and PDP-11 software archive,




 hosting for SIMH site

Van Snyder

IBM 1401 zero footprint bootstraps

Michael Somos

PDP-1 debugging

Hans-Michael Stahl
OS/2 port testing, TERMIOS implementation

Tim Stark

TS10 PDP-10 simulator

Larry Stewart

Initial suggestion for the project

Bill Strecker

Permission to revert copyrights

Chris Suddick

PDP-11 floating point debugging

Ben Supnik

Macintosh timing routine

Bob Supnik

SIMH simulators

Ben Thomas

VMS character-by-character I/O routines

Warren Toomey

PDP-11 UNIX software

Deb Toivonen

DEC documentation

Mike Umbricht

DEC documentation, H316 documentation and schematics

Leendert Van Doorn
PDP-11 UNIX V6 debugging, TERMIOS implementation

Fred Van Kempen
Ethernet code, RK611 emulator, PDP-11 debugging, VAX/Ultrix debugging

Holger Veit

OS/2 socket support

David Waks

PDP-8 ESI-X and PDP-7 SIM8 software

Tom West

Nova documentation

Adrian Wise

H316 simulator, documentation, and software

John Wilson

PDP-11 simulator and software

Joe Young

RP debugging on Ultrix-11 and BSD

Jordi Guillaumes i Pons
Testing and CR11/CD11 fixes

In addition, the following companies have graciously licensed their software at no cost for hobbyist use:

Data General Corporation

Digital Equipment Corporation

Compaq Computer Corporation

Mentec Corporation

The Santa Cruz Operation

Caldera Corporation

Hewlett-Packard Corporation

